Case Study Analysis on Edge Computing

ITAI 3377

Maira Tanweer Chachar

Houston City College

02/07/2026

Table of Contents

Introduction and Objectives	3

Methodology	3

Technology and Implementation	4

Validation and Performance	4

Real-World Applications	4

Challenges and Future Work	4

Personal Evaluation	5

References	6

# Introduction and Objectives

The Liverpool Smart Pedestrians project was developed to address the increasing mobility pressures in Liverpool, NSW, a rapidly growing urban center expecting significant rises in pedestrian and vehicle activity. The primary objective was to design a real-time, privacy-preserving traffic monitoring system that could transform the existing CCTV network into an intelligent sensing infrastructure. The project aimed to detect and track pedestrians, cyclists, and vehicles without transmitting raw video, which allowed the system to comply with strict Australian privacy regulations. These goals directly responded to urban planning challenges such as population growth, congestion, limited mobility data, and the need for evidence-based planning tools (Barthélemy et al., 2019).

# Methodology

The methodology combined community engagement, technical design, and iterative evaluation. Community workshops and consultations with urban planners identified four essential requirements: multimodal detection, privacy compliance, reuse of existing CCTV infrastructure, and scalability. Based on these requirements, the team designed an edge-computing sensor capable of performing object detection and tracking directly on the device. The methodology included hardware prototyping, selection of YOLO V3 and SORT, integration with LoRaWAN and Ethernet, and deployment across twenty locations. The project also considered constraints such as limited bandwidth, strict privacy laws, and the need for reliable performance under varying environmental conditions (Barthélemy et al., 2019).

# Technology and Implementation

The sensor architecture combined specialized hardware with optimized deep-learning software. The NVIDIA Jetson TX2 was selected for its GPU-accelerated performance and low power consumption, making it suitable for continuous edge processing. YOLO V3 was chosen because it provides a strong balance of speed and accuracy and supports multi-scale detection, which is essential for identifying objects at different distances. SORT was used for real-time tracking through Kalman filtering and the Hungarian algorithm. The edge-computing paradigm was central to the system because it reduced bandwidth usage, protected privacy by keeping video on-device, and supported distributed intelligence. These characteristics align with the principles of edge architectures discussed in Module 2 (Ullo & Sinha, 2021).

# Validation and Performance

The system was validated using the Town Centre Dataset and through real-world deployments. The sensor achieved approximately twenty frames per second, which met the requirement for real-time processing. YOLO V3 demonstrated strong detection accuracy for pedestrians and vehicles, although performance decreased for small or heavily occluded objects. SORT maintained stable tracking with minimal identity switches, supported by IoU-based matching and Kalman prediction. System utilization metrics showed efficient GPU usage and manageable CPU load, confirming that the Jetson TX2 could sustain continuous operation under real-world conditions (Barthélemy et al., 2019).

# Real-World Applications

The project demonstrated two significant real-world applications. The first was an indoor emergency evacuation test in which the sensor successfully tracked pedestrian movement and identified flow patterns and bottlenecks. This showed potential for safety analytics and emergency planning. The second was the outdoor deployment across Liverpool’s central business district, where fifteen fixed sensors and five mobile units captured multimodal traffic flows. The data supported urban planners by providing continuous, privacy-safe mobility information. This deployment reflects Module 3 concepts related to sensor networks, edge gateways, and interoperable IoT infrastructures (Ullo & Sinha, 2021).

# Challenges and Future Work

The project encountered several challenges. Occlusion and crowd density reduced detection accuracy. Lighting and weather variations affected image quality. LoRaWAN bandwidth limitations restricted transmission frequency. Re-identification issues occurred when objects left and re-entered the frame. The authors proposed improvements such as enhanced models, better occlusion handling, and integration with additional sensor types (Barthélemy et al., 2019). Since 2019, advancements such as YOLOv7 and YOLOv8, transformer-based vision models, NVIDIA Jetson Orin hardware, 5G connectivity, and improved IoT protocols like MQTT-SN and Matter could significantly enhance accuracy, reduce latency, and expand deployment possibilities (Ullo & Sinha, 2021).

# Personal Evaluation

The Liverpool Smart Pedestrians project is a strong example of how edge-AI can modernize existing infrastructure and support smart city development. Its emphasis on privacy, scalability, and interoperability aligns with contemporary urban technology standards. The project demonstrated that real-time mobility analytics can be achieved without transmitting raw video or installing new cameras. Future improvements could include upgrading to newer AI models, incorporating sensor fusion for low-light environments, and implementing re-identification algorithms to maintain consistent object IDs. Overall, the project provides a practical and forward-looking model for cities seeking to enhance mobility monitoring while maintaining privacy and minimizing costs.

# References

Barthélemy, J., Verstaevel, N., Forehead, H., & Perez, P. (2019). Edge-Computing Video Analytics for Real-Time Traffic Monitoring in a Smart City. Sensors, 19(9), 2048. https://doi.org/10.3390/s19092048

Ullo, S. L., & Sinha, G. R. (2021). Advances in IoT and Smart Sensors for Remote Sensing and Agriculture Applications. Remote Sensing, 13(13), 2585. https://doi.org/10.3390/rs13132585